Rendering - HW2

Result:

- dof-dragons.dgauss.pbrt

- 32 samples

- 512 samples

- dof-dragons.fisheye.pbrt

- 32 samples

- 512 samples

- dof-dragons.telephoto.pbrt

- 32 samples

- 512 samples

- dof-dragons.wide.pbrt

- 32 samples

- 512 samples

Method:

- Collect relation data – initial()

  - Read XXXX.pbrt data

Get by CreateRealisticCamera() method for system arthitecture.

  - Read XXXX.dm file

In initial method - RealisticCamera()

Read file and split by ‘tab’ to get float data.

All store in a vector by lensData data structure.

- Ray tracing – GenerateRay()

  - Generate a ray

Create a ray by sample a point in the near lens and the point in the film in camera space

  - Ray-lens interaction

For each lens face element, from near to far, calculate the intersection between ray and lens, and change the ray origin and direction.

  - Return ray and weight